Perfect. We’ll build this module-wise like an actual project, not like the usual college garbage dump.

Your project is basically:

Predictive Decision Support System for Sustainable Rainwater Harvesting

And your main goal is:

Predict how much rainwater can be collected, how long it can be used, and recommend whether the storage is sufficient.

✅ FINAL PROJECT MODULES

We’ll keep it clean and realistic.

Module 1 – Data Input Module

Module 2 – Rainwater Harvesting Calculation Module

Module 3 – Water Storage & Usage Prediction Module

Module 4 – Decision Support / Recommendation Module

Module 5 – Dashboard / Output Visualization Module

That’s your full project.

Now let’s build each one properly.

## MODULE 1 – DATA INPUT MODULE

This module collects all the required data from the user or dataset.

Purpose

To gather the environmental and usage-related data needed for rainwater harvesting prediction and decision-making.

Inputs Required

1. Monthly Rainfall (mm)

This is the amount of rainfall received each month.

Example:

| Month | Rainfall (mm) |
| --- | --- |
| Jan | 12 |
| Feb | 18 |
| Mar | 25 |
| Apr | 40 |
| May | 95 |
| Jun | 140 |
| Jul | 180 |
| Aug | 165 |
| Sep | 120 |
| Oct | 85 |
| Nov | 30 |
| Dec | 15 |

2. Catchment Area (Roof Area in m²)

This is the surface area from which rainwater is collected.

Example:

- 80 m²

- 100 m²

- 120 m²

3. Runoff Coefficient

This tells how much rainwater can actually be collected.

Different surfaces have different values:

- Concrete roof → 0.8 to 0.9

- Metal roof → 0.85 to 0.95

- Tiled roof → 0.75 to 0.85

Example:

- Runoff coefficient = 0.8

4. Tank Capacity (Litres)

Maximum amount of water that can be stored.

Example:

- 5000 L

- 10000 L

- 15000 L

5. Monthly Water Demand (Litres)

Amount of water used every month.

Example:

| Month | Demand (L) |
| --- | --- |
| Jan | 2500 |
| Feb | 2500 |
| Mar | 2600 |
| Apr | 2700 |
| May | 2800 |
| Jun | 3000 |
| Jul | 3000 |
| Aug | 2900 |
| Sep | 2800 |
| Oct | 2700 |
| Nov | 2600 |
| Dec | 2500 |

Possible Input Sources

You can mention these in report:

- User manual input

- CSV / Excel dataset

- Historical rainfall records

- Regional weather datasets

Output of Module 1

This module outputs a clean structured dataset that will be used by the next modules.

Mini report write-up for Module 1

You can directly use this:

Module 1: Data Input Module
This module is responsible for collecting all the necessary input parameters required for rainwater harvesting prediction. The system accepts monthly rainfall values, rooftop catchment area, runoff coefficient, storage tank capacity, and monthly water demand. These values may be entered manually by the user or loaded from a prepared dataset. The collected data forms the basis for all further calculations and predictions in the system.

## MODULE 2 – RAINWATER HARVESTING CALCULATION MODULE

This module calculates how much water can be collected every month.

Purpose

To estimate the amount of harvestable rainwater based on rainfall and roof characteristics.

Formula Used

[
\text{Harvested Water (Litres)} = \text{Rainfall (mm)} \times \text{Catchment Area (m²)} \times \text{Runoff Coefficient}
]

Why this works

Because:

- 1 mm rainfall on 1 m² = 1 litre of water

So no weird conversion headache needed.

Example Calculation

Suppose:

- Rainfall = 100 mm

- Roof Area = 100 m²

- Runoff coefficient = 0.8

Then:

[
100 \times 100 \times 0.8 = 8000 \text{ litres}
]

So that month can collect 8000 L.

Monthly Harvested Water Table Example

| Month | Rainfall (mm) | Area (m²) | Runoff Coefficient | Harvested Water (L) |
| --- | --- | --- | --- | --- |
| Jan | 12 | 100 | 0.8 | 960 |
| Feb | 18 | 100 | 0.8 | 1440 |
| Mar | 25 | 100 | 0.8 | 2000 |
| Apr | 40 | 100 | 0.8 | 3200 |

Output of Module 2

This module outputs:

- Monthly harvested water

- Annual harvested water

Mini report write-up for Module 2

Module 2: Rainwater Harvesting Calculation Module
This module estimates the quantity of rainwater that can be harvested from the catchment area for each month. The system uses monthly rainfall, catchment area, and runoff coefficient to compute the amount of harvestable water. The calculation is performed using the formula: Harvested Water = Rainfall × Catchment Area × Runoff Coefficient. This module provides the water collection values required for storage and usage prediction.

## MODULE 3 – WATER STORAGE & USAGE PREDICTION MODULE ⭐ (MOST IMPORTANT)

This is your core module.

This is what makes your project actually worth something.

Purpose

To simulate how much harvested water remains in storage after monthly usage and to predict how long it can be used.

Logic Used

At every month:

[
\text{New Storage} = \text{Previous Storage} + \text{Harvested Water} - \text{Water Demand}
]

But:

- if storage exceeds tank capacity → cap it

- if storage goes below 0 → set it to 0

So final logic is:

[
\text{Storage}i = \min(\text{Tank Capacity}, \text{Storage}{i-1} + \text{Harvested}_i - \text{Demand}_i)
]

If result < 0, storage = 0.

What this module predicts

This module helps determine:

- how much water remains every month

- when the tank becomes full

- when the water runs out

- how long the stored water lasts

- which months face shortage

- which months face overflow

Example Table

| Month | Harvested (L) | Demand (L) | Storage at End (L) | Status |
| --- | --- | --- | --- | --- |
| Jan | 960 | 2500 | 0 | Shortage |
| Feb | 1440 | 2500 | 0 | Shortage |
| Mar | 2000 | 2600 | 0 | Shortage |
| Jun | 11200 | 3000 | 8200 | Good |
| Jul | 14400 | 3000 | 10000 | Overflow |
| Aug | 13200 | 2900 | 10000 | Overflow |

Outputs of Module 3

This module gives:

- month-wise stored water

- months where shortage occurs

- months where overflow occurs

- duration of usable stored water

Mini report write-up for Module 3

Module 3: Water Storage and Usage Prediction Module
This module simulates the storage and consumption of harvested rainwater over time. It calculates the monthly water balance by considering the harvested water, tank capacity, and monthly water demand. Based on these values, the system predicts how much water remains available after each month, identifies shortage and overflow conditions, and estimates the duration for which the stored water can be used effectively.

## MODULE 4 – DECISION SUPPORT / RECOMMENDATION MODULE

This is where your system acts “smart”.

Without this, it’s just calculation.

Purpose

To provide useful recommendations based on predicted storage and usage results.

What this module recommends

Based on simulation results, the system can recommend:

1. Whether tank size is sufficient

- If frequent overflow → tank may be too small

- If frequent shortage → tank or collection is insufficient

2. Ideal tank capacity

Suggest better storage size.

3. Water usage sustainability

Whether harvested water can support the demand.

4. Harvesting efficiency

How much collected water is actually utilized.

Sample recommendations

- “Current tank capacity is insufficient for peak rainfall months.”

- “Increase tank size to 12,000 L to reduce overflow loss.”

- “Stored rainwater can support demand for 7 out of 12 months.”

- “The system is moderately sustainable for current demand.”

Decision Rules (simple)

You can literally code simple conditions:

If shortage months > 5:

→ Low sustainability

If shortage months between 2 and 5:

→ Moderate sustainability

If shortage months < 2:

→ High sustainability

Mini report write-up for Module 4

Module 4: Decision Support and Recommendation Module
This module analyzes the results of rainwater harvesting and storage simulation to provide meaningful recommendations to the user. It evaluates whether the current tank capacity and harvested water are sufficient to meet the water demand. Based on shortage and overflow conditions, the system suggests improvements such as increasing storage capacity, optimizing water usage, and assessing the sustainability level of the rainwater harvesting setup.

## MODULE 5 – DASHBOARD / OUTPUT VISUALIZATION MODULE

This is for presentation and project demo.

Even if backend is simple, a good dashboard makes it look 5x better.

Purpose

To display predictions, storage trends, and recommendations visually for easy understanding.

What to show in dashboard

You can include:

1. Monthly Rainfall Chart

Bar chart

2. Monthly Harvested Water Chart

Bar / line chart

3. Storage Level Over Time

Line chart

4. Demand vs Harvested Water

Comparison chart

5. Shortage / Overflow Summary

Pie chart or summary cards

6. Final Recommendation Panel

Text summary

Tools you can use

Best options:

- Power BI (best for your background)

- Python Streamlit

- Excel dashboard

- Matplotlib / Plotly

Mini report write-up for Module 5

Module 5: Dashboard and Output Visualization Module
This module presents the analytical and predictive results in a user-friendly visual format. It displays monthly rainfall, harvested water, storage levels, demand comparison, and sustainability recommendations using charts and summary indicators. The dashboard improves interpretability and helps users understand the effectiveness of the rainwater harvesting system more clearly.

✅ FINAL FLOW OF THE WHOLE SYSTEM

Step 1:

Input rainfall, area, runoff, tank size, demand

Step 2:

Calculate monthly harvested water

Step 3:

Simulate storage and usage over months

Step 4:

Analyze shortages, overflow, and water duration

Step 5:

Give recommendations and show dashboard